Remote Camera Training Manual

Setting Up the Camera

- Set up the tripod first.  Make sure all knobs are tightened, but be sure not to over tighten.  Doing this can damage the screws and cause the unit to be permanently loosened.  Level tripod!

- Attach the shoe to the bottom of the camera.  Be sure not to over tighten the screw.

- Place camera on top of the tripod, locking the shoe into place. Set the knobs to the desired drag for both horizontal & vertical movements, keep the lock on or knobs tight when mounting or releasing camera.

- Insert the battery or hook up the AC power.  If you are using the AC power supply make sure the unit is turned ON.

- Plug in external mics and headphone and test the audio. If  using 1 mic set switch to correct spot on camera so audio will be recorded to both channels if needed.

- If the camera is a MiniDV camera, insert the tape. If it’s card based, insert a memory card or record directly to the camera’s hard drive.

- Take off lens cap or open lens cover w/ lever (on Sony PD-170)

- Look to see if the lens is clean, only clean with a soft microfiber cloth.

- White balance camera if needed.

Manually Focus Your Shot

- Make sure the camera is set to manual focus and not “auto” – most of the time the camera should be on the manual focus setting.

- Zoom all the way in on the subject and manually adjust the focus until it becomes clear.

- Zoom out and set up the shot.

- You will have to manually refocus your shot when the distance between the subject and the camera changes significantly.

Types of Shots

Try not to cut off at the joints, elbow, knee & waist.

- Wide Shot – The camera is zoomed all the way out showing as much of the scene as possible.

- Medium Shot – Upper body of the individual.

- Close Up – Head and shoulders of the individual.

- Extreme Close Up – Zooming all the way in or nearly all the way in on a subject.

Other Terms to Know

- Head Room – How much room is between the top of the head and the TV screen.

- If there is too little head room then the subject will look crowded in the scene.

- If there is too much head room then the subject will look like a talking head.

- Eyes should be about 1/3 from the top

- Nose room- if the talent is facing left or right, make sure there is more space in front of them.

- Pan Left / Right – Pivoting the camera left or right.

- Tilt Up / Down – Pivoting the camera up or down.

- If you will be editing the project before airing it:  Make sure that extra room is left at the beginning & end of each shot and in between segments.  This means letting the camera record for 10 secs or more after the footage needed is captured. Ask talent to wait 4 seconds looking into the camera and then speak.  At the end of their talking tell them to keep looking at the camera for at least 4 seconds and continue recording for another 5 seconds.  This will give you much cleaner footage to edit. Keeping a log of the time on good takes will save a lot of time in post.

Putting the Camera Away

- Take your tape out and label it with the time and title. You should move the safety record tab if you will not be adding any more footage. If you recorded to an SD cord or hard drive, load the footage to the computer, saving it to a file with the project name.

- Make sure the camera is turned off. & the shutter closed or lens cap replaced.

- Unplug mics (XLR connectors have a button to push in to disconnect) Turn off the mic to conserve the battery if it has a switch, some don‘t.

- Do not leave the tripod shoe on the camera or on the tripod.  Put the shoe in the camera case.

- Never leave the camera equipment in the car or trunk for extended times! Do not use in the rain or snow.